PEER NGA-West 2 Flatfile Column Definition and Explanation

This document describes the columns in the PEER NGA W-2 flatfile.  Additional information and documentation is provided in Ancheta et al. [2013, hereafter AEA13] and Seyhan et al. [2014].

Abbreviations used in this document (order of appearance):

Pacific Engineering and Analysis: PE&A; Northern California Seismic Network: NCSN; Southern California Seismic Network: SCSN; Coordinated universal time: UTC; United States Geologic Survey: USGS; California Geologic Survey: CGS; California Strong Motion Instrumentation Program: CSMIP; Ancheta et al. (2013): AEA13; footwall: FW; hanging wall: HW; Uniform Building Code: UBC; National Earthquake Hazards Reduction Program: NEHRP; Resolution of Site Response Issues in the Northridge Earthquake: ROSRINE; US Nuclear Regulatory Commission Regulation: NUREG; Spectral analysis of surface waves: SASW.

Table 1. Finite fault models for earthquakes added in NGA-West2 database.

| Column | Column Name (units) | Description |
| --- | --- | --- |
| A | Record Sequence Number | An arbitrary unique number assigned to each strong-motion record in the flatfile for identification. |
| B | EQID | An arbitrary unique ID assigned to each earthquake for identification. |
| C | Earthquake Name | The common name of earthquake. The naming usually includes the name of the general area or country where earthquake occurred. In case of multiple earthquakes in the same general area/country (for example there are 8 earthquakes in the flatfile that are from Imperial Valley, CA), we used a number to distinguish between these events. Events with a numerical ID are from the NCSN or SCSN earthquake ID. |
| D | YEAR | Year of earthquake. |
| E | MODY | Month and Day (UTC) of earthquake. |
| F | HRMN | Origin time (UTC) of earthquake (Hour and Minute) |
| G | Station Name | The unique name of strong-motion station, which may differ slightly from data provider station name. When it is part of an array, a short phrase is sometimes added to indicate the location of the instrument (for example, "Rio Del Overpass E Ground" and "Rio Del Overpass W Ground"). |
| H | Station Sequence Number | An arbitrary unique sequence number assigned to each strong-motion station for identification. |
| I | Station ID No. | Station ID assigned by data provider (USGS, CGS/CSMIP, etc.). When it is not available, Station ID is assigned a value of "-999". |
| J | Earthquake Magnitude | Moment magnitude of earthquake. When there are multiple reliable estimates of earthquake magnitude, the average value of the reliable estimates is used. |
| K | Magnitude Type | If the listed earthquake magnitude is NOT a moment magnitude, this column identifies the type of magnitude ML = local magnitude MS = surface-wave magnitude Mb = body wave magnitude U = unknown magnitude type (-999) = unknown |
| L | Magnitude Uncertainty: Kagan Model | Magnitude uncertainty assigned using Kagan's model [Kagan, 2002]. |
| M | Magnitude Uncertainty: Statistical | Magnitude uncertainty is taken as the standard deviation of the reliable magnitude estimates. |
| N | Magnitude Sample Size | Number of magnitude estimates used to compute the standard deviation. |
| O | Magnitude Uncertainty: Study Class | Magnitude uncertainty is assigned by PE&A based on the quality of special studies that yielded the magnitude estimates and is defined as: 0.3 --> Older events not well studied 0.2 --> Older events well studied or recent events not well studied 0.1 --> Recent events well studied |
| P | Mo (dyne.cm) | Seismic moment calculated from earthquake magnitude, treating it as a moment magnitude (Mw) regardless of magnitude type; Log10(Mo) = 3/2 * Mw + 16.05 |
| Q | Strike (deg) | Strike angle of the fault plane used to approximate the causative fault surface. 0o <= Strike <= 360o. Convention of fault strike, dip, and rake follows that described in Aki and Richards [1980, p106]. |
| R | Dip (deg) | Dip angle of the fault plane. 0o <= Dip <= 90o. (see note above) |
| S | Rake Angle (deg) | Rake is the angle measured on the fault plane counterclockwise from the reference strike direction to the average slip direction (see graphic for column Q) -180o <= Rake <= 180o |
| T | Mechanism Based on Rake Angle | Mechanism Class Rake Angles --------------------------------------------------------------------------------- Strike - Slip 00 -180 < Rake < -150 -30 < Rake < 30 150 < Rake < 180 --------------------------------------------------------------------------------- Normal 01 -120 < Rake < -60 --------------------------------------------------------------------------------- Reverse 02 60 < Rake < 120 --------------------------------------------------------------------------------- Reverse - Oblique 03 30 < Rake < 60 120 < Rake < 150 --------------------------------------------------------------------------------- Normal - Oblique 04 -150 < Rake < -120 -60 < Rake < -30 |
| U | P-plunge (deg) | This and the next 3 columns list the plunge and trend of the P- and T-axes. P- and T-axes are the maximum (P) and minimum (T) compressive principal stresses given by the fault plane solution. It is suggested that the plunge of the axes may be used to classify fault type, the advantage being that the classification may be more physically based than a simple classification based on rake angle (as the previous column), and furthermore the classification is not dependent on the choice of fault plane. |
| V | P-trend (deg) | (see note above) |
| W | T-plunge (deg) | (see note above) |
| X | T-trend (deg) | (see note above) |
| Y | Hypocenter Latitude (deg) |  |
| Z | Hypocenter Longitude (deg) |  |
| AA | Hypocenter Depth (km) |  |
| AB | Coseismic Surface Rupture: 1=Yes; 0=No; 99=Unknown | Presence or absence of primary surface rupture. |
| AC | Coseismic Surface Rupture (Including Inferred) | This column is mainly an effort to fill the 'Unknown' in the previous column with a value inferred from indirect evidence of surface rupture. |
| AD | Basis for Inference of Surface Rupture | sfdoc = the reference documents the presence or absence of surface faulting sfdis = surface faulting discussed in references locdis = location of earthquake discussed in references M<6 = Magnitude < 6.0, likelihood of existence of surface rupture is small M>7 = Magnitude > 7.0, likelihood of existence of surface rupture is large |
| AE | Finite Rupture Model: 1=Yes; 0=No | If 1, a geometric representation of the ruptured area was developed using observed surface rupture, published slip model(s), aftershock distribution (and time after mainshock), etc. |
| AF | Depth to Top Of Fault Rupture Model | Depth to the top of the finite rupture model (km). Datum is may be ground surface or sea level depending on source. |
| AG | Fault Rupture Length (km) | Total length ("L") of the finite rupture model |
| AH | Fault Rupture Width (km) | Width ("W") of the finite rupture model ( = A / L) |
| AI | Fault Rupture Area (km^2) | Total area ("A") of the finite rupture model |
| AJ | Avg Fault Disp (cm) | The average amount of slip over the ruptured area. It's computed as Mo/(Mu*A*1.0E+10) where Mu=3.58e11. |
| AK | Rise Time (s) | The time required for the completion of slip at a point on the fault plane. When there are multiple estimates of rise time, the average value is used. |
| AL | Avg Slip Velocity (cm/sec) | Avg Slip Velocity = (Avg Fault Displacement) / (Rise time) |
| AM | Static Stress Drop (bars) | Static stress drop is calculated as 7/16*Mo/(A*1.0E+10/pi)^1.5/1.0E+06. |
| AN | Preferred Rupture Velocity (km/sec) | Rupture velocity (Vr) is the speed at which a rupture front moves along the fault during an earthquake. When there are multiple estimates of rupture velocity, the average value is used. |
| AO | Average Vr/Vs | Ratio of rupture velocity (Vr) to shear-wave velocity (Vs) in the source region. When there are multiple estimates, the average value is used. |
| AP | Percent of Moment Release in the Top 5 Km of Crust | This column is calculated from an appropriate slip model. See AEA13 for the slip model used for each earthquake. |
| AQ | Existence of Shallow Asperity: 0=No; 1=Yes | (see note below) |
| AR | Depth to Top of Shallowest Asperity (km) | An asperity is defined by Somerville et al. [1999] as a rectangular region in which the slip exceeds, in a specified way, the slip averaged over the entire fault rupture. If the depth of the top of the shallowest asperity was less than 5 km, the earthquake is classified as a shallow asperity event, and the "Existence of Shallow Asperity" column has a value of 1. If the depth of the top of the shallowest asperity was greater than 5 km, the earthquake is classified as a deep asperity event, and the "Existence of Shallow Asperity" column has a value of 0. |
| AS | Earthquake in Extensional Regime: 1=Yes; 0=No | Extensional regions are regions in which the lithosphere is expanding. Aside from obvious evidence of areal expansion, such as contemporary geodetic measurements and in situ stress measurement, extensional regimes usually present some or all of the following features: a mixture of normal faulting and strike slip earthquakes, recent volcanism, aligned volcanic features, lithospheric thinning, and high heat flow. (Text is excerpted from Spudich et al. [1997].) |
| AT | Fault Name | Name of the causative fault. It is taken from the fault database of the National Seismic Hazard Maps [Frankel et al., 2002] |
| AU | Slip Rate (mm/Yr) | Slip rate on the causative fault. It is taken from the fault database of the National Seismic Hazard Maps [Frankel et al., 2002]. |
| AV | EpiD (km) | Distance from the recording site to the epicenter. |
| AW | HypD (km) | Distance from the recording site to the hypocenter. |
| AX | Joyner-Boore Dist. (km) | Shortest horizontal distance from the recording site to the vertical projection of the rupture on the surface |
| AY | Campbell R Dist. (km) | Shortest distance from the recording site to the seismogenic portion of the ruptured area [Campbell, 1997]. This distance measure assumes that rupture within the near-surface sediment or the shallow portion of fault gouge is non-seismogenic. The depth below which rupture is seismogenic was estimated using updated guidelines from Campbell (personal communication). |
| AZ | RmsD (km) | Root-mean-squared distance |
| BA | ClstD (km) | Closest distance from the recording site to the ruptured fault area |
| BB | Rx (km) | Horizontal distance (km) from top edge of rupture. Measured perpendicular to the fault strike. |
| BC | FW/HW Indicators | This column mimics the FW/HW indicator defined in Abrahamson and Somerville [1996]. hw: site is within hanging wall region fw: site is within the footwall region nu: site is in the neutral region na: not applicable as fault dip is greater than 70 |
| BD | Source to Site Azimuth (deg) | The (finite) source to site direction: It is the angle measured clockwise from the fault strike direction to the direction connecting the site and the site's closest point on the surface projection of top edge of fault. This column can be used an alternative to the FW/HW indicator given previously. |
| BE | X | Somerville et al. [1997] X parameter (non-dimensional units). For multiple-segment faults calculated following a method of Boatwright [2007]. |
| BF | Theta.D (deg) | Somerville et al. [1997]  parameter (degrees). For multiple- segment faults calculated following a method of Boatwright [2007]. |
| BG | SSGA (Strike Slip only) | Somerville et al. [1997] X cos() parameter. For multiple-segment faults calculated following a method of Boatwright [2007]. Calculated for sites within 50km (ClstD) of a strike-slip fault (mechanism = 0). |
| BH | Y | Somerville et al. [1997] Y parameter (non-dimensional units). For multiple-segment faults calculated following a method of Boatwright [2007]. |
| BI | Phi.D (deg) | Somerville et al. [1997]  parameter (degrees). For multiple- segment faults calculated following a method of Boatwright [2007]. |
| BJ | SSGA (Dip Slip) | Somerville et al. [1997] Y cos() parameter (degrees). For multiple-segment faults calculated following a method of Boatwright [2007]. Calculated for sites within 50km (ClstD) on the footwall (FW) or hanging wall (HW) of a dip-slip fault (mechanism not equal to 0). |
| BK | s (km) | s parameter in Spudich and Chiou [2008]. s is the length of fault that ruptures toward site. |
| BL | d (km) | d parameter in Spudich and Chiou [2008]. d is the width of fault that ruptures toward site. |
| BM | c.tilde.prime | The approximate isochrone velocity ratio in Spudich and Chiou [2008]. |
| BN | m5 | Obsolete, no longer used |
| BO | D (km) | D parameter in Spudich and Chiou [2008]. Distance between hypocenter and Xc, the point on the fault plane closest to the site |
| BP | Rfn.Hyp | Amplitude at the site of a fault normal component of a point earthquake source at the event hypocenter. |
| BQ | Rfp.Hyp | Amplitude at the site of a fault parallel component of a point earthquake source at the event hypocenter. |
| BR | Rfn.Clst | Obsolete, no longer used |
| BS | Rfp.Clst | Obsolete, no longer used |
| BT | Rfn.Imd | Obsolete, no longer used |
| BU | T | Generalized T Coordinate at the site, produced using an updated version of the algorithm in Appendix A of Spudich and Chiou [2008] |
| BV | GMX's C1 | First Letter of Geomatrix's Site Classification: Instrument Housing -- Structure Type and Instrument Location. Descriptions included in Table 2. |
| BW | GMX's C2 | Second Letter of Geomatrix's Site Classification: Mapped Local Geology. Descriptions included in Table 3. |
| BX | GMX's C3 | Third Letter of Geomatrix's Site Classification: Geotechnical Subsurface Characteristics. Descriptions included in Table 4. |
| BY | Campbell's GEOCODE | Site Classification defined by Campbell and Bozorgnia [2003], with suggested Vs30 from Wills and Silva [1998] and extended NEHRP site classes as defined by Wills et al. [2000]. Descriptions included in Table 5. |
| BZ | Bray and Rodriguez-Marek SGS | Site Classification defined by Bray and Rodriguez-Marek [1997; personal communication, 2003]. Descriptions included in Table 6. |
| CA | Depth | An assessment of 'soil depth' used in Bray and Rodriguez-Marek SGS. S = alluvium is shallower than 60 m D = alluvium is greater than 60 m |
| CB | Preferred NEHRP Based on VS30 | The preferred NEHRP site class was determined based on the preferred VS30 values. Descriptions included in Table 7. |
| CC | Preferred VS30 (m/sec) | VS30 assignment hierarchy discussed in Section 3.5.1 of AEA13. |
| CD | not used |  |
| CE | Measured/Inferred Class | This column identifies the source of the preferred VS30. The numeric value is related to assignment hierarchy discussed in Section 3.5.1 of AEA13. If assignment is based on a numeric and character (e.g., 2a), it is translated into a numeric value 2.1. |
| CF | Sigma of VS30 (in natural log units) | Uncertainty of VS30. Sigma (standard deviation) values are estimated based on the source of the preferred VS30 value and site category. |
| CG | NEHRP Classification from CGS's Site Condition Map | Extended NEHRP – UBC Site Classification for CA [Wills et al. 2000; Wills, personal communication, 2003]. Descriptions included in Table 8. |
| CH | Geological Unit | Currently this column is populated only for CA sites with information from California Geological Survey (CGS). |
| CI | Geology | A short description of geology from various sources. |
| CJ | Owner | Owner of strong-motion instrument/record. It is not necessarily the most recent owner. |
| CK | Station Latitude | From data provider, when available. |
| CL | Station Longitude | From data provider, when available. |
| CM | STORIES | Number of stories above ground. |
| CN | INSTLOC | Location of instrument in structure. |
| CO | Depth to Basement Rock | Campbell-Bozorgnia definition of sediment depth (text is excerpted from an e-mail written by Ken Campbell to Maury Power, dated 5/19/03). The general criteria used to estimate sediment depth can be described by the following: 1. Set D = 0 for a site categorized as Firm Rock (Hard Rock of Campbell, 1997). Firm Rock has a VS30 of around 817 + 365 m/sec for those sites for which a measured value of VS30 is available. These sites are located primarily in California. This VS30 corresponds approximately to NEHRP site class BC and stiffer according to the classification proposed by CGS (Wills et al., 2000). Geologically, Firm Rock can be defined as pre-Tertiary sedimentary rock and “hard” volcanic deposits, high-grade metamorphic rock, crystalline rock, and the “harder” units of the Franciscan Complex generally described as sandstone, greywacke, shale, chert, and greenstone. 2. Where the depth to Firm Rock is known, set D to that depth (e.g., in the LA Basin where a map showing the depth to basement complex (crystalline rock) is available or in the San Francisco Bay Area where depth to Franciscan or crystalline rock is available). 3. Where the depth to Firm Rock is not known but where basin depth can be inferred from gravity and/or density data, set D to that depth. 4. Where no other information is available, but a local or regional velocity model is available, set D to the depth corresponding to seismic basement (defined as Vp approximately equal to 5.0 km/sec or greater and/or Vs approximately equal to 2.9 km/sec or greater). 5. Where multiple types of information are available, judgment must be applied to determine the best estimate of D, roughly using the order of items 1-4 above as the priority assigned to each type of data. |
| CP | Site Visited | Site visited by geologist or engineer. This is a sparsely populated column; currently only sites in Southern California are populated. |
| CQ | NGA Type | Categories for Vs30 estimation as defined in Borcherdt (personal communication, 2003; 1994, 2002) and Borcherdt and Fumal (2002) 1. Measured value at the station <300 m (a) USGS OFR, ROSRINE, Agbabian; (b) NUREG; (c) SASW; (d) Data gaps in VS30 record 2. Estimate based on velocities measured at nearby sites (< 1500 m distance) in same geologic unit; site visited by geologist 3. Estimate based on velocities measured at site in same geologic unit and judged to have similar materials; site visited by geologist 4. Estimate based on average velocity for the geologic unit; site visited by geologist 5. Estimate based on average velocity for the geologic unit where geologic unit is defined based on large-scale geologic/physical properties map (1:24,000 to 1:100,000 scale) 6. Estimate based on average velocity for the geologic unit where geologic unit is defined based on small-scale geologic map (1:250,000 to 1:750,000 scale) This is a sparsely populated column; currently only sites in Southern California are populated. |
| CR | Age | Geological age of surface material. |
| CS | Grain Size | Grain size of surface material: Aggregate, Coarse, or Fine. |
| CT | Depositional History |  |
| CU | Z1 (m) | Depth to Vs=1.0 km/sec. This column is populated with stations within the 3-D velocity models of Southern California named CVM--S4 is the version 4 model developed by Harold Magistrale and others (http://scec.usc.edu/scecpedia/CVM--S), N. Cal (Aagaard), and the Eel River basin [Graves, 1994], with the addition of information from the PE&A Profile Data Base of depths to 1 km/sec, 1.5 km/sec and 2.5 km/sec when these velocities were measured at the site. |
| CV | Z1.5 (m) | Depth to Vs = 1.5 km/sec (see note for CU) |
| CW | Z2.5 (m) | Depth to Vs = 2.5 km/sec (see note for CU) |
| CX | Z1 (m) | Depth to Vs=1.0 km/sec. This column is populated with stations within the 3-D velocity models of S. Cal using CVM--H11.1.0 is the version 11.1.0 Harvard model developed by John Shaw and others (http://scec.usc.edu/scecpedia/CVM--H), N. Cal (Aagaard), and the Eel River basin [Graves, 1994], with the addition of information from the PE&A Profile Data Base of depths to 1 km/sec, 1.5 km/sec and 2.5 km/sec when these velocities were measured at the site. |
| CY | Z1.5 (m) | Depth to Vs = 1.5 km/sec (see note for CX) |
| CZ | Z2.5 (m) | Depth to Vs = 2.5 km/sec (see note for CX) |
| DA | Depth to Franciscan Rock (km) | This column is populated only for stations in the Bay Area. |
| DB | Basin | Name of the sedimentary basin. This column and the next 6 columns contain basin parameters defined and used in Joyner [2000] and Somerville et al. [2002]. These columns are sparsely populated. |
| DC | h (m) | Depth to basement. |
| DD | hnorm (m) | = h / Rsbe. |
| DE | Rsbe (m) | Closest distance from the station to the basin edge. |
| DF | Rcebe (m) | Perpendicular distance from the station to the basin edge. |
| DG | Rebe (m) | Distance from the epicenter to the basin edge along a line between the epicenter and the station. |
| DH | Rsbe1 (m) | Distance from the station to the basin edge along a line between the epicenter and the station. |
| DI | File Name (Horizontal 1) | Directory name and file name of time history data files. Note that file name is made up of station abbreviation and instrument orientation. If the orientation is -999 then that component did not record the event. |
| DJ | File Name (Horizontal 2) | (Same as above) |
| DK | File Name (Vertical) | (Same as above) |
| DL | H1 component azimuth (degrees) | Azimuth (deg) |
| DM | H2 component azimuth (degrees) | Azimuth (deg) |
| DN | Type of Recording | A - Analog D - Digital A large portion of this column is not populated. |
| DO | Instrument Model | A large portion of this column is not populated. |
| DP | PEA Processing Flag | PEA: Acceleration time history record was processed using PE&A's standard processing procedure starting with the Volume 1 (uncorrected) time series; #: Acceleration time history record is directly from the Volume 2 (corrected) accelerogram as received from the data provider (pass-through records), except for the ChiChi aftershocks records. In the case of ChiChi aftershocks, equivalents to the Volume 2 records were created by the NGA-West1 project. |
| DQ | Type of Filter | This column lists the type of filter used by PE&A or data providers (mainly USGS and CSMIP) to remove noise at long and short periods. O - Ormsby A - Acausal Butterworth C - Causal Butterworth |
| DR | npass | Number of passes of filter. |
| DS | nroll | This column lists the 'nroll' parameter of the Butterworth filter. -999 when not applicable (Ormsby filter). |
| DT | HP-H1 (Hz) | Corner frequency of the high-pass filter of component H1. 1. When component unavailable, corner frequency is -999. 2. If filter was not applied, corner frequency is 0 3. The definition of corner frequency varies with the filter type. In the case of a Butterworth filter, the corner frequency is the frequency at which the filter response is at -3db of the maximum response. In the case of an Ormsby filter, the corner frequency is the beginning point of the transition frequency band. |
| DU | HP-H2 (Hz) | Corner frequency of the high-pass filter of component H2. (see note for HP-H1) |
| DV | LP-H1 (Hz) | Corner frequency of the low-pass filter of component H1. (see note for HP-H1) |
| DW | LP-H2 (Hz) | Corner frequency of the low-pass filter of component H2. (see note for HP-H1) |
| DX | Factor | This column ("Factor") gives the ratio of the lowest usable frequency ("LUF") to the corner frequency ("HP") of the high-pass filter. The recommended lowest usable frequency is the frequency above which spectra from high-pass filtered data are relatively unaffected by the filter. For convenience, "LUF" is evaluated in terms of "Factor". "Factor" is determined according to the filter type and order of the filter. 1. When a Butterworth filter is used, "LUF" is taken as the frequency at which filter response is -0.5db down from the maximum response (or 94% of the maximum). "Factor" is then determined from the number of passes ("npass") and "nroll" of the Butterworth filter. 2. For Ormsby filter, "Factor" is 1. 3. When filter is not applied (blank "HP"), empirical relationship is used to determine "LUF" and the value for "Factor" becomes irrelevant. |
| DY | Lowest Usable Freq - H1 (Hz) | This column ("LUF") is the product of "HP-H1" and "Factor", except when "HP-H1" is -999 or 0. When "HP-H1" is -999, "LUF" is again -999. When "HP-H1" is 0, "LUF" is determined from an empirical relationship between "LUF" and earthquake magnitude and type of recording. |
| DZ | Lowest Usable Freq - H2 (Hz) | (see note above) |
| EA | Lowest Usable Freq - Ave. Component (Hz) | This column is the recommended lowest usable frequency for the average horizontal component. It is taken as the larger of the two previous columns. If any of the two previous columns is -999 (i.e., one or both of the two horizontal components did not record the event), this column is left blank. |
| EB | PGA (g) | This and the next 114 columns list the peak acceleration (PGA), peak velocity (PGV), peak displacement (PGD), and pseudo spectral accelerations (5%-damped) at 111 periods. 1. Listed ground-motion value may be the as recorded motions, RotDnn from Boore [2010], or GMRotI50 from Boore et al. [2006]. The name of the tab and file will indicate the type. 2. Pseudo spectral acceleration and absolute acceleration are in units of g. |
| EC | PGV (cm/sec) |  |
| ED | PGD (cm) |  |
| EE | T0.010S |  |
| …….. | …….. |  |
| IK | T20.000S |  |
| IL | Ave Strike (deg) | Local strike-parallel direction is the fault strike direction averaged over a 20 km (or less) stretch of fault segment beginning at the closest point on the fault and extending towards the epicenter. |
| IM | TYPE(CRjb = 0) | Fault classification using a CRjb distance = 0 km. C1 are class 1 events and C2 are class 2 events. C2 - xxxx is a class 2 event along with the associated class 1 event EQID number. The 'f' following C1 indicates that is it a class 1 foreshock to the 'xxxx' EQID listed. See Section 2.6 of AEA13 and Wooddell and Abrahamson [2014] for more details. |
| IN | CRjb | CRjb for the class 2 events in column IM (km) |
| IO | TYPE (Rjb = 2) | Fault classification using a CRjb distance = 2 km. Additional detail in column IM above. |
| IP | CRjb | CRjb for the class 2 events in column IO (km) |
| IQ | TYPE (Rjb = 5) | Fault classification using a CRjb distance = 5 km. Additional detail in column IM above. |
| IR | CRjb | CRjb for the class 2 events in column IQ (km) |
| IS | TYPE (Rjb = 10) | Fault classification using a CRjb distance = 10 km. Additional detail in column IM above. |
| IT | CRjb | CRjb for the class 2 events in column IS (km) |
| IU | TYPE (Rjb = 20) | Fault classification using a CRjb distance = 20 km. Additional detail in column IM above. |
| IV | CRjb | CRjb for the class 2 events in column IU (km) |
| IW | TYPE (Rjb = 40) | Fault classification using a CRjb distance = 40 km. Additional detail in column IM above. |
| IX | CRjb | CRjb for the class 2 events in column IW (km) |
| IY | U | Generalized U Coordinate at the site, produced using an updated version of the algorithm in Appendix A of Spudich and Chiou [2008]. |
| IZ | IDPv4 | An update of the Isochrone Directivity Parameter, produced using the algorithm in Appendix A of Spudich and Chiou [2008]. |
| JA | XCI () | Directivity Parameter, B. Rowshandel using the methodology presented in the Spudich et al. [2013,2014] |
| JB | XCI1 (1) | Directivity Parameter, B. Rowshandel using the methodology presented in the Spudich et al. [2013,2014] |
| JC | Damping (%) | Damping values used are 0.5, 1, 2, 3, 5, 7, 10, 15, 20, 25, 30 |
| JD | RotD fractile | Fractiles used are 00, 50, 100. |
| JE | Instrument Natural Frequency (Hz) | This and the following two columns (JG-JI) are instrument information for stations used in the California small-to-moderate magnitude data processing. |
| JF | Instrument Damping | As decimal. |
| JG | Instrument Type | ‘A’ for acceleration time history. ‘V’ for a velocity time history. |
| JH | Quality Flag | Judgment on the quality of the time history recording. See section 6.8 of AEA13. |
| JI | Spectral Quality Flag | Judgment on the quality of the spectra. See section 6.8 of AEA13. |
| JJ | Late S-trigger | See Table 9 for flag explanation. |
| JK | Late P-trigger | See Table 9 for flag explanation. |
| JL | Idirectivity | Pulse-like (1) or non-pulse-like (0) indicator determined using the method presented in Shahi and Baker [2014] |
| JM | Tp | The pseudo-period of the extracted pulse using the method presented in Shahi and Baker [2014] |
| JN | Ry | Horizontal distance (km) from the top edge of the rupture, measure along fault strike. |

Table 2. Geomatrix first letter: Instrument structure type.

| GMX First Letter | Instrument Structure Type |
| --- | --- |
| I | Free-field instrument or instrument shelter. Instrument is located at or within several feet of the ground surface, and not adjacent to any structure. |
| A | One-story structure of lightweight construction. Instrument is located at the lowest level and within several feet of the ground surface. |
| B | Two- to four-story structure of lightweight construction, or very large tall) one-story warehouse-type building. Instrument is located at the lowest level and within several feet of the ground surface. Also, Haviland Hall and Mt. Hamilton Lick Observatory are two-story buildings founded on rock. The instrumentation is in a vault with the instrument founded on rock at these two BDSN stations. |
| A,B | Used for small generally lightweight structures for which we cannot determine the number of stories from the available information. These sites generally have COSMOS site code 4 which defines a reference station described as either a 1- or 2-story, small, light building. This classification is mainly used in the small-moderate magnitude data set. |
| C | One- to four-story structure of lightweight construction. Instrument is located at the lowest level in a basement and below the ground surface. |
| D | Five or more story structure of heavy construction. Instrument is located at the lowest level and within several feet of the ground surface. |
| E | Five or more story structure of heavy construction. Instrument is located at the lowest level in a basement and below the ground surface. |
| F | Structure housing instrument is buried below the ground surface about 1-2 m, at a shallow depth. e.g., tunnel or seismic vault (e.g., U. S. Array design) but shallow embedment (use 'T' for deeper embedments or 'V' for deeply embedded vaults, both not considered "free-field") |
| I,F | These sites generally have COSMOS site code 3 for which the sensors have been buried/set in ground at shallow or near surface depths (e.g., the U. S. Array station design). This classification is mainly used in the small-moderate magnitude western and EUS data sets. |
| G | Structure of light or heavyweight construction, instrument not at lowest level. |
| H | Dam either earth or concrete (station at toe of embankment or on abutment). |
| J | Concrete Dam structural instrumentation (none in data base). |
| K | Near a one-story structure of lightweight construction. Instrument is located outside on the ground surface, within approximately 3 m from the structure. |
| L | Near a two- to four-story structure. Instrument is located outside on the ground surface, within approximately 6 m of the structure. |
| M | Near a two- to four-story structure with basement. Instrument is located outside on the ground surface, within approximately 6 m of the structure. |
| N | Near a five- to eight-story structure. Instrument is located outside on the ground surface, within approximately 10 m of the structure. |
| O | Near a five- to eight-story structure with basement. Instrument is located outside on the ground surface, within approximately 10 m of the structure. |
| P | Castle of masonry construction, massive 1-3 stories |
| Q | Associated with a structure, size of structure is not known |
| S | Associated with a structure and in the basement, size of structure is not known. |
| T | Associated with a deep tunnel, e.g., a) L'Aquila - Parking: Pleistocene terrace above a pedestrian tunnel on the edge's slope of the terrace, nearby structure to the station is a car park. b) Various BDSN stations (e.g., WDC, WENL, YBH). |
| U | Il Moro is on an embankment between two roads and retaining walls. |
| V | Deeply embedded seismic vault |
| W | Structural response e.g., roof, penstock, etc. (e.g., CSMIP 23732, San Bernardino - Devil's Canyon Penstock) |
| Z | At depth in borehole or missile silo. |

Table 3. Geomatrix second letter: mapped local geology.

| GMX Second Letter | Mapped local geology sedimentary or metasedimentary rocks |
| --- | --- |
| H | Holocene (Recent) Quaternary (< 11,000 ybp). |
| Q | Pleistocene Quaternary (< 1.8my bp). |
| P | Pliocene Tertiary (< 5my bp). |
| M | Miocene Tertiary (< 24my bp). |
| O | Oligocene Tertiary (< 34my bp). |
| E | Eocene Tertiary (< 55my bp). |
| L | Paleocene Tertiary (< 65my bp). |
| K | Cretaceous (< 144my bp). |
| F | Franciscan Formation (Cretaceous/Late Jurassic). |
| J | Jurassic (< 206my bp). |
| T | Triassic (<248my bp). |
| Z | Permian or older (> 248my bp). Igneous or meta-igneous: |
| V | Volcanic (extrusive). |
| N | Intrusive. |
| G | Granitic. |

Table 4. Geomatrix third letter: geotechnical subsurface characteristics.

| GMX Third Letter | Geotechnical subsurface characteristics |  |
| --- | --- | --- |
| A | Rock. Instrument on rock (Vs > 600 mps) or < 5m of soil over rock. |  |
| B | Shallow (stiff) soil. Instrument on/in soil profile up to 20m thick overlying rock. |  |
| C | Deep narrow soil. Instrument on/in soil profile at least 20m thick overlying rock, in a narrow canyon or valley no more than several km wide. |  |
| D | Deep broad soil. Instrument on/in soil profile at least 20m thick overlying rock, in a broad valley. |  |
| E | Soft deep soil. Instrument on/in deep soil profile with average Vs < 150 mps. |  |
| CD | Deep soil site, valley dimensions not known. |  |
| For many Italian events, the third letter is followed by a "*". This denotes classifications inferred from local geology information provided with the unprocessed data |  |  |
| GMX Third Letter (number) |  | Central Weather Bureau (CWB) of Taiwan Site Categories |
| 1 |  | Hard site. |
| 2 |  | Medium site. |
| 3 |  | Soft soil site. |

Table 5. GEOCODE site classifications from Campbell and Bozorgnia [2003].

| Campbell's GEOCODE | Description |
| --- | --- |
| A | Firm Soil: Holocene; recent alluvium, alluvial fans, undifferentiated Quaternary deposits., Vs30 298±92 m/sec; NEHRP D |
| B | Very Firm Soil: Pleistocene; older alluvium or terrace deposits. Vs30 368±80 m/sec; NEHRP CD |
| C | Soft Rock: Sedimentary rock, soft volcanic deposits of Tertiary age, “softer” Franciscan, low grade metamorphic rocks such as mélange, serpentine, schist. Vs30 421±109 m/sec; NEHRP CD |
| D | Firm Rock: Older sedimentary rock and hard volcanic deposits, high grade metamorphic rock, crystalline rock, “harder” Franciscan Vs30 830±339 m/sec; NEHRP BC |
| E | Shallow Soils (10 m deep) |
| F | Extremely soft or loose Holocene age soils such as beach sand or recent flood-plain, lake, swamp estuarine, and delta deposits. |

Table 6. Site Classification defined by Bray and Rodriguez-Marek [1997].

| Bray and Rodriguez-Marek SGS | Description | Comments |
| --- | --- | --- |
| A | Hard rock | Hard, strong, intact rock; Vs30 1500 m/sec |
| B | Rock | Most “unweathered” California rock cases (Vs30 760 m/sec or < 6 m of weathered rock or soil). |
| C | Weatherd soft rock/ shallow stiff soil | Weathered rock zone > 6 m and < 60 m (Vs30 > 360 m/s increasing to > 700 m/sec); Soil depth < 60 m |
| D | Deep stiff soil | Soil depth > 60 m and < 3 m of soft soils |
| E | Soft clay | Thickness of soft clay > 3 m |
| F | Special | Potentially Liquefiable Sand or peat: Holocene loose sand with high water table (zw <= 6 m) or organic peat. |
| U | Unknown Conditions | Unknown Conditions |

Table 7. NEHRP site classification [BSSC, 1994]

| NEHRP site classification | Average shear-wave velocity to a depth of 30 m |
| --- | --- |
| A | > 1500 m/sec |
| B | 760 m/sec- 1500 m/sec |
| C | 360 m/sec– 760 m/sec |
| D | 180 m/sec – 360 m/sec |
| E | < 180 m/sec |

Table 8. Extended NEHRP – UBC Site Classification for California [Wills et al., 2000]

| Site classification | UBC (extended) Site Classification |
| --- | --- |
| B | Plutonic and metamorphic rocks, most volcanic rocks, coarse sedimentary rocks of Cretaceous age and older. |
| BC | Franciscan Complex rocks of the Transverse Ranges which tend to be more sheared, Cretaceous siltstones, or mudstones. |
| C | Franciscan mélange and serpentine, sedimentary rocks of Oligocene to Cretaceous age or coarse-grained sedimentary rocks of younger age. |
| CD | Sedimentary rocks of Miocene and younger age, unless formation is notably coarse-grained, Plio-Pleistocene alluvial units, older (Pleistocene) alluvium, some areas of coarse younger alluvium. |
| D | Younger (Holocene) alluvium. |
| DE | Fill over bay mud in the San Francisco Bay Area, fine-grained alluvium and estuarine deposits elsewhere along the coast. |
| E | Bay mud and similar intertidal mud. |

Table 9. Summary of late P-wave and S-wave trigger flags.

| Flag | Description |
| --- | --- |
| d | Digital with pre-event memory, did not record the first P-wave onset. In the digital case we are confident that we recorded the largest amplitude (provided, later peaks are > the nominal trigger level of 0.005 g, which depends on instrument, array, etc.) |
| ? | Analog recording that probably recorded largest amplitudes on the vertical component but the P-wave onset not recorded, OR probably recorded largest amplitudes on the horizontal component(s) for the S-wave. |
| Y | Late trigger, probably did not record the largest vertical amplitude, The characteristic diagnostic is that the largest amplitude is at start of recording on the vertical component. |
| DNP | Did not process the vertical component (e.g. late trigger) |
| DNR | Did not record the vertical component. |
| D | The largest amplitude on the component was recorded as well as the onset of motion |
| ^ | P-wave onset recorded because the analog instrument was already recording an earlier event |
| PT | Recordings were not processed by PE&A from the volume 1. This symbol is used with other symbols (e.g. “D PT”, “? PT”) |
| ! | NA |
| * | NA |

# References

Ancheta, T.D., Darragh, R.B., Stewart, J.P., Seyhan, E., Silva, W.J., Chiou, B.S.J., Woodell, K.E., Graves, R.W., Kottke, A.R., Boore, D.M., Kishida, T. and Donahue, J.L., 2013. PEER NGA-West 2 database, PEER Report 2013/03, Pac. Earthq. Eng. Res. Ctr, Berkeley, CA.

Abrahamson, N.A., Somerville, P.G., 1996. Effects of the hanging wall and footwall on the ground motions recorded during the Northridge earthquake, Bull. Seismol. Soc. Am., 86, S93–S99.

Aki, K., and Richards, P., 1980. Quantitative Seismology, Freeman, New York.

Boatwright, J., 2007. The persistence of directivity in small earthquakes, Bull. Seismol. Soc. Am., 97, 1850-1861.

Boore, D.M., Watson-Lamprey, J., and Abrahamson, N.A., 2006. Orientation-independent measures of ground motion. Bull. Seismol. Soc. Am. 96, 1502-1511.

Boore, D.M. 2010. Orientation-independent, nongeometric-mean measures of seismic intensity from two horizontal components of motion. Bull. Seismol. Soc. Am. 100, 1830-1835.

Borcherdt, R.D., and Fumal, T.E., 2002. Shear-wave velocity compilation for Northridge strong-motion recording sites, U.S. Geological Survey Open File Report 2002-107.

Bray, J.D., and Rodriguez-Marek, A., 1997. Geotechnical site categories proceedings, First PEER-PG&E Workshop on Seismic Reliability of Utility Lifelines, San Francisco, CA, Au- gust.

BSSC, 1994. NEHRP recommended provisions for seismic regulations for new buildings, Part 1-Provisions, FEMA 222A, Federal Emergency Management Agency.

Campbell, K.W., 1997, Empirical near-source attenuation relationships for horizontal and vertical components of peak ground acceleration, peak ground velocity, and pseudo-absolute acceleration response spectra, Seismol. Res. Lett, 68, 154–179.

Campbell, K.W., and Bozorgnia, Y., 2003. Updated near-source ground-motion (attenuation) relations for the horizontal and vertical components of peak ground acceleration and accel- eration response spectra, Bull. Seismol. Soc. Am., 93, 314–331.

Frankel, A.D., Petersen, M.D., Mueller, C.S., Haller, K.M., Wheeler, R.L., Leyendecker, E.V., Wesson, R.L., Harmsen, S.C., Cramer, C.H., Perkins, D.M., Rukstales, K.S., 2002. Documentation for the 2002 Update of the National Seismic Hazard Maps, U.S. Geological Survey Open File Report 02-420.

Graves, R.W., 1994. Simulating the 3D basin response in the Portland and Puget Sound re- gions from large subduction zone earthquakes, USGS Award: 1434-93-G-2327, Annual Technical Report.

Joyner, W.B., 2000, Strong motion from surface waves in deep sedimentary basins, Bull. Seismol. Soc. Am., 90, S95–S112.

Kagan, Y.Y., 1993, Statistics of characteristic earthquakes, Bull. Seismol. Soc. Am., 83, 7-24.

Shahi, S.K., Baker, J.W., 2014. NGA-West2 models for ground-motion directionality, Earthq. Spectra, This volume (in review).

Somerville, P.G., Smith, N.F., Graves, R.W., and Abrahamson, N.A., 1997. Modification of empirical strong ground motion attenuation relations to include the amplitude and duration effects of rupture directivity, Seismol. Res. Lett., 68, 199-222.

Somerville, P.G., Irikura, K., Graves, R.W., Sawada, S., Wald, D., Abrahamson, N.A., Iwasaki, Y., Kagawa, T., Smith, N.F., Kowada, A., 1999.Charaterizing crustal earthquake slip models for the prediction of strong ground motion, Seismol. Res. Lett., 70, 59-80.

Somerville, P.G., Collins, N., Graves, R., and Pitarka, A., 2002. Development of an engineer- ing model of basin generated surface waves, Proceedings of SMIP03 Seminar on Utilization of Strong-Motion Data, 1–21.

Spudich, P., Fletcher, J.B., Hellweg, M., Boatwright, J., Sullivan, C., Joyner, W.B., Hanks, T.C., Boore, D.M., McGarr, A., Baker, L.M., Lindh, A.G., 1997. A new predictive relation for earthquake ground motions in extensional tectonic regimes, Seismol. Res. Lett., 68, 190-198.

Spudich, P., and Chiou, B. S. J., 2008. Directivity in NGA earthquake ground motions: analysis using isochrone theory, Earthq. Spectra,, 24, 279–298.

Spudich P., Bayless J.R., Baker J.K., Chiou B.S.J., Rowshandel B., Shahi S.K., Somerville P., 2013. Final Report of the NGA-West 2 directivity working group, PEER Report 2013/09, Pac. Earthq. Eng. Res. Ctr, Berkeley, CA.

Spudich P., Rowshandel B., Baker J.K., Shahi S.K., 2014. Overview and comparison of the NGA-West2 directivity models. Earthq. Spectra, This issue (in review).

Wills, C.J., Petersen, M., Bryant, W.A., Reichle, M., Saucedo, G.J., Tan, S., Taylor, G., and Treiman, J., 2000. A site-conditions map for California based on geology and shear-wave velocity, Bull. Seismol. Soc. Am., 90, S187-S208.

Wills, C.J., and Silva, W.J., 1998. Shear wave velocity characteristics of geologic units in Cali- fornia, Earthq. Spectra, 14, 533–556.

Wooddell, K.E. and Abrahamson, N.A., 2014. Classification of mainshocks and aftershocks in NGA-West 2, Earthq. Spectra, This issue (in review).